Hardware Design and Lab: Lab1

111060013 EECS 26' 劉祐廷

- Basic

- Basic Question 1: Block Diagram

- Basic Question 3: The Difference Between Full Adder and Half Adder

The most significant difference between them is that a half adder can only deal with the situation without carry in; however, a full adder can handle the situation with carry in.

- Advanced: Decode and execute

- Block Diagram: Basic Modules

- Explanation: Basic Modules

First of all, I drew the truth table of Universal Gate to design my own Not module and Copy module. And then I combined a Not module and an Universal Gate to create And module. After that, I make a Nand module with an And module and a Not module. The reason why I design Nand module before designing other modules (ex: Or, Xor) is that I have designed several modules consist with only nand gates in Basic Question 1. By designing out the Nand module first, I could design other modules more easily by only replacing all nand gates with Nand modules.

- Block Diagram: Advanced Modules

- Explanation: Some Useful Modules

- Majority: Count the carry out

- Half_adder: Deal with 1-bit addition without carry in

- Full_Adder: Deal with 1-bit addition with carry in

- CompareEQ: Compare two 4-bit data whether they are equal or not and output rd

- CLT_1bit: Compare two 1-bit data rs and rt and output 1 if rs < rt

- CLT_4bits: Compare two 4-bit data rs and rt and output 1 if rs < rt

- CompareLT: Combine the result from CLT_4bits and output rd

- LeftShift: Use four Copy modules to shift rs

- RightShift: Use four Copy modules to shift rt

- BitwiseAnd: Use four And modules to realize it

- BitwiseOr: Use four Or modules to realize it

- BitwiseInvt: Use four Not modules to realize it

- Add: Use a Half_Adder module and three Full_Adder modules to make this ripple carry adder

- Sub: Use a BitwiseInvt module and a Add module to get the negation of rt and then use a Add module to get rd = rs + (-rt)

- Decode_And_Execute: Use the modules mentioned above to deal with eight situation and then use a Mux_8to1 module to choose which result should be output

- Testbench

- Advanced: 8-bit carry-lookahead (CLA) Adder

- Block Diagram

- Explanation

  - sourse: https://chi_gitbook.gitbooks.io/personal-note/content/addition.html

CarryCounter0: count c1 by c0, p and g

CarryCounter1: count c2 by c0, p and g

CarryCounter2: count c3 by c0, p and g

CarryCounter3: count c4 by c0, p and g

PGS_Generator: count p, g and s (sum) by a, b and c (carry in)

CLA_Generator_2bit: Combine the data generated by CLA_Generator_4bit

CLA_Generator_4bit: generate carry for each bit simultaneously

Carry_Look_Ahead_Adder_8bit: add two 8-bit data by carry-look-ahead method

- Testbench

Most of this module is same as the Crossbar_2x2_4bit module in 1. Crossbar_2x2_4bit. The difference is that this module connects to two Fanout_1to2_4bits modules in order to link to two LED on FPGA board for each output.

To test this design, I programed it on FPGA board and turn the switch on and off to check every situation of input and output is correct.

- An exhaustive testbench design

In Lab 1, I've learned the difference of thinking ways between software design and hardware design. Hardware design is more like putting blocks together while software design is more like dealing with some different events. Through this lab, I've got more familiar to Vivado and FPGA board. I hope that these experiences can help me learn well in the following classes.

- Decode and execute (FPGA)

- Block Diagram

- Explanation

After counting rd by the Decode_And_Execute module, rd would be passed into an encode module called Segment7_Display to transform to correct value to control 7-segment display on FPGA board.

- Discussion