Hardware Design and Lab: Lab4

111060013 EECS 26' 劉祐廷

Catalog

1.	Basic Question:

Many-to-one LFSR and One-to-many LFSR………………P3

2.	Advanced Question:

Content-addressable memory (CAM) design…...………….P4

3.	Advanced Question:

Scan Chain Design…….……………………….…………..…...P6

4.	Advanced Question:

Built-in Self Test……………………………………….……..P8

5.	Advanced Question:

Built-in Self Test FPGA…………………………………....P10

6. Advanced Question:

Mealy Sequence Detector…………………………………P15

6.	What I Have Learned…...................................................P16

- Basic Question

- Many-to-one LFSR

- State Transition Diagram

Figure 1.1

- Basic Question

If we reset the DFFs to 8'd0, then DFFs will remain 8'd0 because (DFF[1] ^ DFF[2]) ^ (DFF[3] ^ DFF[7]) is 0. Therefore, out will be a constant 1'b0.

- One-to-many LFSR

- State Transition Diagram

Figure 1.2

- Basic Question

If we reset the DFFs to 8'd0, then DFFs will remain 8'd0 because DFF[7] XOR any other bit is 0. Therefore, out will be a constant 1'b0.

- Advanced Question: Content-addressable memory (CAM) design

- Block Diagram

Figure 2.2

mem

Figure 2.1

isHit[15:0]

Figure 2.3

hit

Figure 2.4

dout

- Explanation

In this problem, I defined a reg called isHit[15:0] as Figure 2.1 to save the hit status. If the i-th bit in isHit equal to din, then isHit[i] will be set to 1'b1. Otherwise it will be set to 1'b0. Figure 2.2 shows how I design the memory. If ren is 1'b0 and wen is 1'b1, then din will be written into mem[addr]. Otherwise, mem[addr] will not change. As mention above, isHit == 16'b0 means that there is no hit condition happened. Therefore, I designed hit as it shows in Figure 2.3. According to the specification, if hit == 1'b1, we should output the largest address of the bits that is hit. So I designed the circuit as it shows in Figure 2.4 to realize this function.

- Testbench

Figure 2.5 wave form 1

First, I use the input data from lecture slide to test if there is something wrong or not. As the result showed in Figure 2.5, we can see that everything works correctly. And then I also add some extra testcases to check if the design work correctly. As Figure 2.6 shows, it seems that it works correctly.

Figure 2.6 wave form 2

- Advanced Question: Scan Chain Design

- Block Diagram

Figure 3.1

SDFF

Figure 3.2

Scan_Chain_Design (Left Part)

Figure 3.3

Scan_Chain_Design (Right Part)

- Explanation

In this problem, the lecture slides provide a complete design. Thus I realize the circuit by Verilog according to the Figure 3.1, Figure 3.2, and Figure 3.3. Note that 4-bit Multiplier is designed by behavioral-level code.

- Testbench

Figure 3.4

Wave form

In this problem, I found that it is difficult to judge the results with the naked eyes. Therefore, as I've learned form Lab2, I defined a signal called err to help me check the result. In testbench, the input sequence will be stored in input_arr[7:0] first. And then it will be scan bit by bit into the DFFs. After input 8 bits of data, it will count multi_ans[7:0] as input_arr[7:4] * input_arr[3:0] and then input_ans will be set to the next testcase. After scan_en pulled up again, the testbench will check the output bit by bit compare to multi_ans. If detecting any errors, err will be pulled up. Note that err will be reset to 1'b0 while scan_en is 1'b0. In this way, I can check the output more efficiently and precisely.

- Advanced Question: Built-in Self Test

- Block Diagram

Figure 4.1 Many_To_One_LFSR

Figure 4.2 Built_In_Self_Test

- Explanation

In this problem, I only have to combine to modules I 've designed in the previous problems. Figure 4.1 shows how I designed Many_To_One_LFSR. Figure 4.2 shows how I connect these two modules together.

- Testbench

Figure 4.3 wave form

In this design, testcases are generated by Many_To_One_LFSR. It seems that the method of how I designed in the testbench of Scan_Chain_Design is risky because there are some chances that I have some wrong cases in my testbench. Therefore, I write a code in C++ to simulate the testcases and check the results one by one. And every result seems right.

Figure 4.4

C++ code for simulating the testcases

- Advanced Question: Built-in Self Test FPGA

- Block Diagram

Figure 5.1

The whole design of this problem

Figure 5.2

Debounce

Figure 5.3

One_Pulse

Figure 5.4

Clock_Devider_SSD

Figure 5.5

SSD_Decoder

Figure 5.6

Built_In_Self_Test

Figure 5.8

mux_b

Figure 5.7

mux_a

Figure 5.10

mux_out

Figure 5.9

mux_in

Figure 5.11

Many_To_One_LFSR

Figure 5.12

SDFF

Figure 5.13

Scan_Chain_Design (Left Part)

Figure 5.14

Scan_Chain_Design (Left Part)

- Explanation

According to the specification, I divided the circuit into different modules as Figure 5.1 shows. In order to work properly on FPGA boards, divided clock is needed. Most of the modules above are modified from the designs of the previous problems by adding a mux which selected by dclk before connect into DFFs. However, after testing on the FPGA board, I noticed that there is some chance that the DFFs will go through two states while I only press the button one time. To solve this problem, I use a divided clock to control Debounce module as Figure 5.15 shows. In this way, it seems the condition mentioned above doesn't happen again while I was testing it on the FPGA board.

Figure 5.15

- Advanced Question: Mealy Sequence Detector

- State Diagram

Figure 6.1

- Explanation

The problem requires that the mealy machine should redetect every 4 bits. Therefore, I define a state called start, which means that there is no bit input. And then I define some states as Figure 6.1 shows. The states can be classified into four different groups by how many bits are pushed in.

- No input: start

- One-bit input: 0, 1

- Two-bit input: 01, 10, 11, 2nd fail

- Three-bit input: 100, 111, 011, 3rd fail

If the mealy machine detects a wrong input, the state will change into 2nd fail or 3rd fail, depending on the number of the bits that have been pushed in. For example, if the mealy machine detects the wrong bit while input the second bit, the state will be changed into 2nd fail.

- What I Have Learned

Most of modules are provided in the lecture slides, thus I spent much less time designing modules. However, it is difficult to design a testbench in this lab. In order to check the results, I used the method that I've learned from Lab2 and also wrote a C++ program to generate testcases correctly to help me debug. In this lab, I've learned that how to combined the skills that I've learned before to solve problems. I hope that I can use these skills more flexible in the future. In addition, I encountered a strange condition while I was doing the FPGA problem. The are some chances that if I press the dclk button once, the state will change twice. In order to solve this problem, I consulted my classmate who has study Logic Design Lab last semester. She told me that if clock is too fast, Debounce will have some chances that works unproperly. Usually, Debounce should use a 100Hz clock. To prevent from the same bug, I will take her advice and implement it in future problems.